PROJECT SPECIFICATION: ADJUSTABLE POMODORO TIMER

Official Engineering Requirements Blueprint & Technical Reference Manual

This document establishes the architecture, electrical properties, user experience definitions, and system logic layout for the custom Arduino Nano Pomodoro Timer.

1. PROJECT DIRECTORY & WORKSPACE LAYOUT

To support proper tracking and version control, all firmware and technical hardware assets must conform strictly to the standardized directory landscape structured below:

Pomodoro_Timer/
├── 📂 hardware/
│   ├── 📂 datasheets/         <-- Physical component pinout & spec sheets
│   └── 📂 enclosure/          <-- Physical measurement specs and structural CAD 
├── 📂 firmware/
│   └── 📂 src/
│       └── main.ino           <-- Core embedded execution C++ firmware asset
└── 📄 project_spec.docx       <-- This comprehensive engineering master print

2. HARDWARE ARCHITECTURE & BILL OF MATERIALS (BOM)

All components must comply with operational current constraints and pin allocations to avoid micro-controller bus conflict or thermal overload.

| Component Class | Functional Scope & Purpose | Electrical Profile / Notes | Datasheet |
| --- | --- | --- | --- |
| Arduino Nano (AZ-Nano V3-Board with ATMEGA328 CH340) | Central computing processing unit. Executes the state-machine loop and monitors I/O. | ATmega328P based. Core execution draw: ~50mA. 5V VCC input | AZ Delivery Website Hardware\Datasheets\ArduinoNano-datasheet.pdf |
| MAX7219 8x32 Display | Alphanumeric digit array layout and progress visualizations. Multi-matrix design. | 5V VCC input. 1320mA max input when all LEDs lit at max brightness. Operational ~800mA | Hardware\Datasheets\MAX7219.pdf |
| KY-040 Rotary Encoder | Primary manual data entry component. Rotation for changing numbers, press for confirmation/cancelling | Utilizes hardware interrupts for snappy feel | Hardware\Datasheets\KY-040 Rotary Encoder.pdf |
| 10µF Electrolytic Capacitor | Decoupling capacitor mounted at display bus to eliminate transient voltage ripple from LED switching | Polarized. Filters high-current switching demand spikes. MUST be parallel to matrix LED display | TBA |
| 5V 1.5A DC Adapter (+ screw terminal adapter) | Primary 5V power via external regulated power supply. | Screw-terminal configuration directly bridging to 5V rail. Possibility of not using screw terminal by cutting wire end and soldering directly to board. | N/A |
| Passive Piezo Buzzer | Generates audible warnings at state boundaries. | Driven via micro-controller PWM pin and NPN transistor at 5V. Operating ~40-80mA | N/A |
| S8050 (NPN Transistor) | Regulate current draw from CPU | Buzzer ground to collector, CPU to base with 1kΩ resistor, emitter to GND | Hardware\Datasheets\SS8050-117753.pdf |
| 1kΩ Resistor | Protect CPU and NPN transistor from high current draw | Placed in series between CPU and base leg of transistor | N/A |
| 10kΩ Resistor | Act as voltage divider for photoresistor | Placed after photoresistor with CPU reading between photoresistor and this resistor | N/A |
| Photoresistor | Allow for auto-dimming feature between three states: Bright, normal, dark | Connected to 5V VCC in series with the 10kΩ resistor to act as voltage divider | Hardware\Datasheets\GL5537-Photoresistor.pdf |

3. PERIPHERAL BUS PIN ALLOCATIONS

The following hardware mapping dictates connections directly linking peripherals across the shared micro-controller backplane:

| Peripheral Module | Module Interface Pin | Arduino Pin Mapping | Bus Type & Execution Notes |
| --- | --- | --- | --- |
| MAX7219 Matrix Array | DIN (Data In) | D11 | Hardware SPI MOSI Line (Fixed) |
| MAX7219 Matrix Array | CS (Chip Select) | D10 | Dedicated Digital Output (Active Low) |
| MAX7219 Matrix Array | CLK (Clock) | D13 | Hardware SPI Serial Clock Output (Fixed) |
| KY-040 Rotary Interface | CLK (Encoder Step) | D2 | Hardware Interrupt Pin 0 (Trigger: FALLING) |
| KY-040 Rotary Interface | DT (Encoder Direction) | D4 | Digital Input (Sampled on CLK Interrupt) |
| KY-040 Rotary Interface | SW (Push Button Switch) | D5 | Digital Input with Active Internal Pull-up Resistor |
| 1kΩ Resistor | To NPN base pin | D6 | Timer Counter 2 Output (PWM Tone Modulation) |
| 5V 1.5A DC Adapter | Positive (+) | 5V | Provide power to CPU |
| 5V 1.5A DC Adapter | Ground (-) | GND | Provide closed circuit for 5V and GPIO pins |
| Photoresistor | Ground (-) | D14 | Hardware ADC 0; Placed between photoresistor and 10kΩ resistor |

4. SYSTEM LOGIC & STATE MACHINE ARCHITECTURE

The firmware executes an explicit Finite State Machine (FSM). The system processes inputs through a sequential configuration routine (Setup Phase) before cycling endlessly through operational states (Active Phase).

Note: An interactive structural diagram of the sequential configurations and cyclical state boundaries is embedded in the primary project_spec.pdf visual companion build. Below is the structural state reference map:

Phase A: Setup Menu Sequence

STATE_SET_WORK: Display 'WRK:XX'. Rotary encoder adjustments increment/decrement target work focus sessions (1-90 mins). A single button click registers parameters and locks memory.

STATE_SET_SHORT: Display 'SHT:XX'. Rotary adjustments alter short breaks (1-15 mins). Single click advances to final parameter block.

STATE_SET_LONG: Display 'LNG:XX'. Sets long resting limits (0-30 mins). Setting the numerical value to '00' natively skips long execution loops. Single click branches to active run routines.

Phase B: Active Operating Loop

STATE_WORK_RUNNING: Focus countdown window active. Bottom-row scanline updates systematically while vertical session indicators remain steadily illuminated.

- OR

STATE_WORK_PAUSED: Halts countdown. Pause shows up on screen to show timer has been paused.

- OR simply flash current time/text

STATE_BREAK_RUNNING: Performs automated baseline checking against current completed intervals to dynamically activate Short or Long break limits.

- OR

- OR

- OR

- OR

STATE_BREAK_PAUSED: Halts countdown. Pause shows up on screen to show timer has been paused.

- OR simply flash current time/text

STATE_ALARM: Triggered at boundaries. Actively routes specific audio wave intervals to physical output pin while throwing alternating blinking alerts. Requires manual click acknowledging.

5. UI LAYOUT DESIGN & VISUAL REAL ESTATE MAP

The MAX7219 matrix platform hosts an 32x8 coordinate grid

Columns 6-31 (Main Matrix Layer): Dedicates a 7-pixel height layer for numeric time text formatting (MM:SS) or alphabetic parameter setup designations (WRK, SHT, LNG).

Column 0-4 (Session and Mode): Shows up to four pips on the bottom two rows to mark the four quarters until a long rest. Lightning bolt (or other) icon indicates work timer, coffee mug (or other) icon indicates break timer

6. SAFETY, POWER & SOFTWARE STABILIZATION PARAMETERS

Display Brightness Ceiling: Array lighting intensity is hard-capped within software boundaries at an operating level of 12 out of 15. This ceiling guarantees that absolute worst-case power spikes remain safely under ~800mA, leaving ample safety margin for the 1.5A adapter.

Total Max Current: 800mA (LEDs) + 80mA (Buzzer) + 50mA (CPU) +

Switch Debounce Filtering: Rotary encoder button inputs utilize a fixed 50ms time-differential lockout parameter, neutralizing structural mechanical contact bounce.